# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply, Delivery, Installation, Commissioning and Operator and Maintenance Training for One (1) Hydraulic Crawler Excavator with Hydraulic Breaker for Re-use and Management of Construction and Demolition Waste	p 1 /…

Publication reference: RE-CONSTRUCT/IPA/2026/SUP/T02

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Excavation Machine - with following technical specifications |  |  |  |
| 1.1 | The excavator shall be equipped with: - Four-stroke turbocharged diesel engine - Minimum four cylinders - Liquid cooling system - Emission standard: Tier IV Final / Stage IV or higher |  |  |  |
| 1.2 | Mobility: - Hydro powered tracks/crawler made of steel - Hydraulic braking - Travelling speed, min. 2 km/h - Transport length with standard dipper maximum 7000mm |  |  |  |
| 1.3 | Excavation capacity: - Digging depth, mm, min. 4,500 - Max Dump Height, mm, min. 5,000 - Standard digging Bucket with min. 4 teeth and min. capacity of 0.20 m3. Excavator, bucket and hydraulic breaker shall be supplied fully assembled, commissioned and ready for operation. - Dozer blade min. 1800 mm wide and 300 mm high. - Movement of the blade min. 350 mm up and down. Lift up/down hydraulic operation |  |  |  |
| 1.4 | Other specifications: - Multifunction operator display including at least: - engine coolant temperature - engine oil pressure warning - fuel level - hour meter - service indicator - warning lamps - Hydraulic pump flow: minimum 150 l/min at 2,200 rpm - Tank capacity, litters Minimum 60 L - 4 pillar ROPS, sunshade and Falling Objects Roof Protection, (complying with applicable ISO standards) - Pallets of the Excavator Chain (Steel tracks) - Standard tool kit necessary for routine maintenance shall be included. - Warning reversing buzzer acoustic and rotating amber beacon, 1 min. - 2 kg fire extinguisher 2kg Powder, CE certified - First aid kit complying with national road safety requirements - Turning circle - wall to wall, m minimum 2500mm radius (+/- 250mm) - Operating weight, kg, min. 8,500 - Hydraulic connections compatible with the hydraulic breaker specified as mandatory additional equipment under Section 1.5. |  |  |  |
| 1.5 | Additional equipment - Hydraulic Breaker - Min. working flow 60-100 l/min - Min. working pressure 120- 170 bar - Impact rate min. 400-800 bpm - Tool diameter min. 75 mm Suitable for the excavator supplied. Compatible hydraulic connections. Complete mounting kit including hydraulic hoses, couplings and all mounting accessories required for immediate operation. The hydraulic breaker shall be supplied installed, connected and ready for operation. |  |  |  |
| 1.6 | Other requirements: The supplier shall ensure: - authorised service partner in North Macedonia - availability of spare parts - qualified service technicians - spare parts available for at least five years after delivery - maximum response time: 48 hours. Response time shall be measured from the moment the Contractor receives the written notification from the Contracting Authority. - Minimum warranty 12 months or 1,500 operating hours whichever occurs first. Warranty shall include: - Labour - replacement parts - travel costs - diagnostics - software updates at no additional cost. |  |  |  |
| 1.7 | Documentation User Manual: The supplier must provide comprehensive user manuals, including maintenance schedules and - Spare Parts Catalogue - Maintenance Manual - Service Schedule - CE Declaration of Conformity - Certificate of Origin - Warranty Certificate. All documentation shall be supplied in English. User, Operation and Safety Manuals shall additionally be supplied in Macedonian. |  |  |  |
| 1.8 | Delivery shall include: - transport - unloading - commissioning - operational testing - Delivery inspection - Provisional Acceptance - operator training - maintenance training - demonstration of hydraulic breaker - complete documentation - completed Delivery Inspection Record, Commissioning Record, Operational Test Record and Training Attendance Record, in accordance with Annex VI - Acceptace Records Package. The excavator shall be delivered fully assembled, fuelled, commissioned and ready for operation. |  |  |  |
| 1.9 | Technical Offer Requirements - Manufacturer's brochures - Technical data sheets - CE Declaration of Conformity - Proposed commissioning methodology. - Proposed operational testing methodology. - Statement confirming compatibility between the excavator, hydraulic breaker and hydraulic connections. - Statement confirming that the excavator will be supplied fully assembled, commissioned and ready for operation. - Statement confirming compliance with every mandatory technical requirement of Annex II. - Preventive maintenance schedule for the first 1,500 operating hours. - Recommended consumables list. - Photographs of the offered model - Manufacturer's catalogue - Warranty statement - List of local service centres - Spare parts availability statement - Delivery schedule Training programme |  |  |  |

Failure to submit sufficient technical documentation enabling the Evaluation Committee to verify compliance with the mandatory technical requirements of Annex II may result in the rejection of the tender. General statements such as "Compliant", "Yes", or references without clearly identifying the offered model and configuration shall not be considered sufficient.